E1 : S’il n’y a pas d’arrivée d’eau pendant plus de 4 minutes et si le pressostat n’a pas basculé et le lave vaisselle s’arrête. Le programme signalera le problème par une alarme sonore durant 30 secondes et le code erreur E1 s’affichera sur le Display. Après l’alarme sonore, la pompe de vidange se mettra en route. On peut simuler ce code erreur en bloquant l’arrivée d’eau
E2 : Si la pompe de vidange tourne pendant 4 minutes, le pressostat (niveau bas) n’est donc pas détecté par le programme  et le lave vaisselle s’arrête. Le programme signalera le problème par une alarme sonore durant 30 secondes et le code erreur E2 s’affichera sur le Display . On peut simuler ce code erreur en bloquant la vidange
E3 : Si la résistance ne chauffe pendant 30 minutes, la température n’est pas détecté par la sonde et le lave vaisselle s’arrête. Le programme signalera le problème par une alarme sonore durant 30 secondes et le code erreur E3 s’affichera sur le Display . On peut simuler ce code erreur en débranchant la résistance
E4 : Si le micro-interrupteur de la sécurité anti-fuite est activé, et le lave vaisselle s’arrête. Le programme signalera le problème par une alarme sonore durant 30 secondes et le code erreur E4 s’affichera sur le Display . Après l’alarme sonore, la pompe de vidange se mettra en route On peut simuler ce code erreur en activant la sécurité anti-fuite
E6 : Si l’électronique détecte une température importante dans le circuit des sondes de sécurité, le lave vaisselle s’arrête. Le programme signalera le problème par une alarme sonore durant 30 secondes et le code erreur E6 s’affichera sur le Display . Après l’alarme sonore, la pompe de vidange se mettra en route
E7 : Si l’électronique détecte un court circuit dans les sondes de sécurité, le lave vaisselle s’arrête. Le programme signalera le problème par une alarme sonore durant 30 secondes et le code erreur E7 s’affichera sur le Display . Après l’alarme sonore, la pompe de vidange se mettra en route
	SYMPTOM
	CHECK FOR THE FOLLOWING
	REEDY

	ERROR1(E1)
	The water supply is turned off
	Turn the water supply on

	
	The pipe was snapped
	Put the pipe in the right way

	
	Low water pressure
	Minimum water pressure of 0.04Mpa

	
	Defective water fill valve
	Replace the water fill valve

	
	Obstructed water fill valve or hose
	Disassemble and clean the water fill valve and hose

	
	Damaged or defective wiring
	Repair the wiring

	
	Defective pressure switch(not for 9319A)
	Replace the pressure switch

	
	Heavy water usage elsewhere in
	 Use dishwasher when

	
	home.
	water usage is at a minimum.

	
	Defective the electric control
	Replace the electric control

	ERROR2(E2)
	The sump or the filter was obstructed
	Clear the sump and filter

	
	Damaged or defective drain pump
	Replace the drain pump

	
	Drain hose was obstructed or snapped
	Put the hose in the right way

	
	Defective pressure switch(not for 9319A)
	Replace the pressure switch

	
	Damaged or defective wiring
	Repair the wiring

	
	Defective the electric control
	Replace the electric control

	ERROR3(E3)
	The voltage too low
	Minimum voltage of 198V

	
	Damaged the thermostat 
	Replace the thermostat

	
	Damaged or defective wiring
	Repair the wiring

	
	The NTC thermistor was damaged or not in the right position 
	Replace the thermistor or make it in the right position

	
	Damaged the heating element
	Replace the heating element

	
	Defective the electric control
	Replace the electric control

	ERROR4(E4)
	The sump or the filter was obstructed
	Clear the sump and filter

	
	Soap suds leak from dishwasher
	Refer to owners manual

	
	Damaged the sump/pump/tub/drain pump
	Repair or replace the sump /pump /tub/drain pump

	
	Loose hose clamps
	Tighten or replace hose clamps

	
	Loose heater element
	Tighten heater element mounting nuts

	
	Loose sump
	Tighten the sump

	
	Loose axe of the lower spray arm
	Tighten the axe

	
	Loose softener
	Tighten the softener

	
	Damaged or defective wiring
	Repair the wiring

	
	Defective the electric control
	Replace the electric control

	
	The wiring be wrong connected with the microswitch after repair
	Right connected with the microswitch

	
	Defective microswitch
	Replace the microswitch

	
	The float too high or something make the microswitch set 
	Repair the float and microswitch

	ERROR6 
	The NTC thermistor was damaged or not in the right position 
	Replace thermistor or make it in the right position

	
	The temperature is lower than -20°
	Temperature should be higher than -20°

	
	Damaged or defective wiring
	Repair  wiring

	
	Defective electric control
	Replace  electric control

	ERROR7
	 NTC thermistor was damaged 
	Replace  thermistor 

	
	 temperature is higher than 120°
	The temperature should be below than 120°

	
	Damaged or defective wiring
	Repair  wiring

	
	Defective  electric control
	Replace  electric control


